# Untitled

| Report date | Oct 15, 2025 10:26:52 AM |
| --- | --- |

Contents

1.	Global Definitions

1.1.	Parameters 1

2.	Component 1

2.1.	Definitions

2.2.	Geometry 1

2.3.	Materials

2.4.	Turbulent Flow, k-ε

2.5.	Mesh 1

3.	Study 1

3.1.	Stationary

3.2.	Solver Configurations

4.	Results

4.1.	Data Sets

4.2.	Plot Groups

Global Definitions

| Date | Sep 14, 2025 11:59:33 PM |
| --- | --- |

Global settings

| Name | Venturi.mph |
| --- | --- |
| Path | C:\Users\USER PC\Desktop\MEE 321\Task 6\VENTURI\Venturi.mph |
| COMSOL version | COMSOL 5.3 (Build: 223) |
| Unit system | SI |

Used products

COMSOL Multiphysics

CFD Module

Parameters 1

Parameters

| Name | Expression | Value | Description |
| --- | --- | --- | --- |
| D | 42[mm] | 0.042 m | Diameter of pipe, inlet and out |
| L_in | 180[mm] | 0.18 m | Inlet length |
| L_out | 110[mm] | 0.11 m | Outlet length |
| D_t | 15[mm] | 0.015 m | Throat diameter |
| L_t | 40[mm] | 0.04 m | Throat length |
| U | 0.011[m/s] | 0.011 m/s | Inlet velocity |
| x0 | 0[mm] | 0 m |  |
| y0 | 0[mm] | 0 m |  |
| xt | 220[mm] | 0.22 m |  |
| yt | 13.5[mm] | 0.0135 m |  |
| x1 | 300[mm] | 0.3 m |  |
| y1 | 0[mm] | 0 m |  |

Component 1

| Date | Jan 9, 2025 3:09:59 AM |
| --- | --- |

Component settings

| Unit system | SI |
| --- | --- |
| Geometry shape order | automatic |

Definitions

Coordinate Systems

#### Boundary System 1

| Coordinate system type | Boundary system |
| --- | --- |
| Tag | sys1 |

Coordinate names

| First | Second | Third |
| --- | --- | --- |
| t1 | n | to |

Geometry 1

Geometry 1

Units

| Length unit | m |
| --- | --- |
| Angular unit | deg |

Geometry statistics

| Description | Value |
| --- | --- |
| Space dimension | 2 |
| Number of domains | 1 |
| Number of boundaries | 12 |
| Number of vertices | 12 |

Rectangle 1 (r1)

Position

| Description | Value |
| --- | --- |
| Position | {x0, y0} |

Size

| Description | Value |
| --- | --- |
| Width | L_in |
| Height | D |

Rectangle 2 (r2)

Position

| Description | Value |
| --- | --- |
| Position | {xt, yt} |

Size

| Description | Value |
| --- | --- |
| Width | L_t |
| Height | D_t |

Rectangle 3 (r3)

Position

| Description | Value |
| --- | --- |
| Position | {x1, y1} |

Size

| Description | Value |
| --- | --- |
| Width | L_out |
| Height | D |

Bézier Polygon 1 (b1)

Polygon segments

| Description | Value |
| --- | --- |
| Control points | {{0.18, 0.22, 0.22, 0.18}, {0.042, 0.0285, 0.0135, 0}} |
| Degree | {1, 1, 1} |
| Weights | {1, 1, 1, 1, 1, 1} |
| Type | Solid |

Bézier Polygon 2 (b2)

Polygon segments

| Description | Value |
| --- | --- |
| Control points | {{0.3, 0.26, 0.26, 0.3}, {0.042, 0.0285, 0.0135, 0}} |
| Degree | {1, 1, 1} |
| Weights | {1, 1, 1, 1, 1, 1} |
| Type | Solid |

Union 1 (uni1)

Compose

| Description | Value |
| --- | --- |
| Keep interior boundaries | Off |

Materials

Water

Water

Selection

| Geometric entity level | Domain |
| --- | --- |
| Selection | Domain 1 |

Material parameters

| Name | Value | Unit |
| --- | --- | --- |
| Dynamic viscosity | eta(T[1/K])[Pa*s] | Pa·s |
| Density | rho(T[1/K])[kg/m^3] | kg/m³ |

Basic Settings

| Description | Value |
| --- | --- |
| Dynamic viscosity | eta(T[1/K])[Pa*s] |
| Ratio of specific heats | 1.0 |
| Electrical conductivity | {{5.5e-6[S/m], 0, 0}, {0, 5.5e-6[S/m], 0}, {0, 0, 5.5e-6[S/m]}} |
| Heat capacity at constant pressure | Cp(T[1/K])[J/(kg*K)] |
| Density | rho(T[1/K])[kg/m^3] |
| Thermal conductivity | {{k(T[1/K])[W/(m*K)], 0, 0}, {0, k(T[1/K])[W/(m*K)], 0}, {0, 0, k(T[1/K])[W/(m*K)]}} |
| Speed of sound | cs(T[1/K])[m/s] |

Functions

| Function name | Type |
| --- | --- |
| eta | Piecewise |
| Cp | Piecewise |
| rho | Piecewise |
| k | Piecewise |
| cs | Interpolation |

eta

Cp

rho

k

cs

Turbulent Flow, k-ε

Used products

COMSOL Multiphysics

CFD Module

Turbulent Flow, k-ε

Selection

| Geometric entity level | Domain |
| --- | --- |
| Selection | Domain 1 |

Equations

Settings

| Description | Value |
| --- | --- |
| Discretization of fluids | P1 + P1 |
| Value type when using splitting of complex variables | {Real, Real, Real, Real, Real, Real, Real, Real, Real, Real, Real, Real, Real} |
| Compressibility | Incompressible flow |
| Turbulence model type | RANS |
| Turbulence model | k - ε |
| Wall treatment | Wall functions |
| Include gravity | Off |
| Reference temperature | 293.15[K] |
| Reference pressure level | 1[atm] |
| Streamline diffusion | On |
| Crosswind diffusion | On |
| Streamline diffusion | On |
| Crosswind diffusion | On |
| Isotropic diffusion | Off |
| Isotropic diffusion | Off |
| Turbulence model parameter | 1.44 |
| Turbulence model parameter | 1.92 |
| Turbulence model parameter | 0.09 |
| Turbulence model parameter | 1.0 |
| Turbulence model parameter | 1.3 |
| Law of the wall constant | 5.2 |
| Turbulence model parameter | 0.41 |
| Use pseudo time stepping for stationary equation form | On |
| CFL number expression | Automatic |
| Velocity scale | 1[m/s] |
| Length scale factor | 0.035 |

Variables

| Name | Expression | Unit | Description | Selection |
| --- | --- | --- | --- | --- |
| spf.Tref | 293.15[K] | K | Reference temperature | Domain 1 |
| spf.dz | 1 | m | Thickness | Domain 1 |
| spf.pref | 1[atm] | Pa | Reference pressure level | Domain 1 |
| spf.pA | p+spf.pref | Pa | Absolute pressure | Domain 1 |
| spf.hasWF | 0 |  | Help variable | Boundaries 1–12 |
| spf.Ceps1 | 1.44 | 1 | Turbulence model parameter | Domain 1 |
| spf.Ceps2 | 1.92 | 1 | Turbulence model parameter | Domain 1 |
| spf.C_mu | 0.09 | 1 | Turbulence model parameter | Domain 1 |
| spf.sigmak | 1 | 1 | Turbulence model parameter | Domain 1 |
| spf.sigmaeps | 1.3 | 1 | Turbulence model parameter | Domain 1 |
| spf.B | 5.2 | 1 | Law of the wall constant | Domain 1 |
| spf.sigmaw | 0.2 | 1 | Smoothing parameter | Domain 1 |
| spf.kappav | 0.41 | 1 | Turbulence model parameter | Domain 1 |
| spf.localCFLvalue | 3+if(niterCMP>=10,1.3^min(-10+niterCMP,7),0)+if(niterCMP>=35,9*1.3^min(-35+niterCMP,9),0)+if(niterCMP>=65,90*1.3^min(-65+niterCMP,9),0) |  |  | Domain 1 |
| spf.locCFL | CFLCMP | 1 | Local CFL number | Domain 1 |
| spf.time_step_inv | max(sqrt(emetric_spatial(u,v)*2^gmg_level^2),spf.mu/(spf.rho*(0.25*spf.l_mix_lim)^2)) | Hz | Inverse time step | Domain 1 |
| spf.nx | dnx | 1 | Normal vector, x component | Boundaries 1–12 |
| spf.ny | dny | 1 | Normal vector, y component | Boundaries 1–12 |
| spf.nz | 0 | 1 | Normal vector, z component | Boundaries 1–12 |
| spf.nxmesh | dnxmesh | 1 | Normal vector, x component | Boundaries 1–12 |
| spf.nymesh | dnymesh | 1 | Normal vector, y component | Boundaries 1–12 |
| spf.nzmesh | 0 | 1 | Normal vector, z component | Boundaries 1–12 |

Weak expressions

| Weak expression | Integration order | Integration frame | Selection |
| --- | --- | --- | --- |
| -spf.rho*nojac(spf.time_step_inv/spf.locCFL)*(ep-nojac(ep))*test(ep) | 2 | Spatial | Domain 1 |
| -spf.rho*nojac(spf.time_step_inv/spf.locCFL)*(k-nojac(k))*test(k) | 2 | Spatial | Domain 1 |
| spf.rho*nojac(spf.time_step_inv/spf.locCFL)*(-(u-nojac(u))*test(u)-(v-nojac(v))*test(v)) | 2 | Spatial | Domain 1 |

Fluid Properties 1

Fluid Properties 1

Selection

| Geometric entity level | Domain |
| --- | --- |
| Selection | Domain 1 |

Equations

Settings

| Description | Value |
| --- | --- |
| Mixing length limit | Automatic |
| Density | From material |
| Dynamic viscosity | From material |
| Temperature | User defined |
| Temperature | 293.15[K] |

Properties from material

| Property | Material | Property group |
| --- | --- | --- |
| Density | Water | Basic |
| Dynamic viscosity | Water | Basic |

#### Variables

| Name | Expression | Unit | Description | Selection |
| --- | --- | --- | --- | --- |
| spf.k_nn | max(k,0) | m²/s² | Regularized turbulent kinetic energy | Domain 1 |
| spf.rho | subst(material.rho,spf.fp1.minput_temperature,spf.Tref,spf.pA,spf.pref) | kg/m³ | Density | Domain 1 |
| spf.mu | subst(material.mu,spf.fp1.minput_temperature,spf.Tref) | Pa·s | Dynamic viscosity | Domain 1 |
| spf.muinit | spf.mu | Pa·s | Dynamic viscosity | Domain 1 |
| spf.rhoinit | subst(spf.rho,spf.fp1.minput_pressure,1[atm]) | kg/m³ | Density | Domain 1 |
| spf.l_mix_lim | 2*spf.lref | m | Mixing length limit | Domain 1 |
| spf.gammaT | nojac(spf.C_mu*spf.rho*spf.k_nn/spf.muT) | 1/s | Turbulence help variable | Domain 1 |
| spf.tauT | 1/spf.gammaT | s | Turbulence time scale | Domain 1 |
| spf.ep_nn | max(ep,0) | m²/s³ | Regularized turbulent dissipation rate | Domain 1 |
| spf.muT | nojac(max(spf.rho*spf.lsubstar*sqrt(spf.k_nn),0.5*spf.mu)) | Pa·s | Turbulent dynamic viscosity | Domain 1 |
| spf.ep_pos | max(ep,eps) | m²/s³ |  | Domain 1 |
| spf.Sxx | ux | 1/s | Strain rate, xx component | Domain 1 |
| spf.Syx | 0.5*(vx+uy) | 1/s | Strain rate, yx component | Domain 1 |
| spf.Szx | 0 | 1/s | Strain rate, zx component | Domain 1 |
| spf.Sxy | 0.5*(uy+vx) | 1/s | Strain rate, xy component | Domain 1 |
| spf.Syy | vy | 1/s | Strain rate, yy component | Domain 1 |
| spf.Szy | 0 | 1/s | Strain rate, zy component | Domain 1 |
| spf.Sxz | 0 | 1/s | Strain rate, xz component | Domain 1 |
| spf.Syz | 0 | 1/s | Strain rate, yz component | Domain 1 |
| spf.Szz | 0 | 1/s | Strain rate, zz component | Domain 1 |
| spf.AbsS | sqrt(spf.Sxx^2+spf.Sxy^2+spf.Sxz^2+spf.Syx^2+spf.Syy^2+spf.Syz^2+spf.Szx^2+spf.Szy^2+spf.Szz^2) | 1/s | Absolute strain rate | Domain 1 |
| spf.rF | sqrt(2)/(3*max(spf.AbsS,eps)) | s | Help variable | Domain 1 |
| spf.rFInv | 3*spf.AbsS/sqrt(2) | 1/s | Help variable | Domain 1 |
| spf.lcore | min(spf.C_mu*spf.k_nn^1.5/spf.ep_pos,sqrt(spf.k_nn)*spf.rF) | m | Help variable | Domain 1 |
| spf.lsubstar | nojac(min(spf.lcore,spf.l_mix_lim)) | m | Limited mixing length | Domain 1 |
| spf.nuT | spf.muT/spf.rho | m²/s | Turbulent kinematic viscosity | Domain 1 |
| spf.linSCk | -spf.rho*spf.gammaT | kg/(m³·s) | Linear source term coefficient, k-equation | Domain 1 |
| spf.linSCeps | -spf.rho*spf.Ceps2*spf.gammaT | kg/(m³·s) | Linear source term coefficient | Domain 1 |
| spf.PCore | 2*ux^2+uy*(uy+vx)+vx*(uy+vx)+2*vy^2 | 1/s² | Turbulent kinetic energy source term | Domain 1 |
| spf.Pk | nojac(max(spf.muT*spf.PCore,0)) | W/m³ | Turbulent kinetic energy source term | Domain 1 |
| spf.Peps | nojac(max(spf.Ceps1*spf.gammaT*spf.muT*spf.PCore,0)) | Pa/s² |  | Domain 1 |
| spf.kinit | (100*spf.muinit/(spf.rhoinit*spf.l_mix_lim))^2 | m²/s² | Turbulent kinetic energy | Domain 1 |
| spf.epinit | 10*spf.C_mu*spf.kinit^1.5/spf.l_mix_lim | m²/s³ | Turbulent dissipation rate | Domain 1 |
| spf.epsilon_p | 1 | 1 | Porosity | Domain 1 |
| spf.Fst_tensorxx | 0 | N/m² | Surface tension force, xx component | Domain 1 |
| spf.Fst_tensoryx | 0 | N/m² | Surface tension force, yx component | Domain 1 |
| spf.Fst_tensorzx | 0 | N/m² | Surface tension force, zx component | Domain 1 |
| spf.Fst_tensorxy | 0 | N/m² | Surface tension force, xy component | Domain 1 |
| spf.Fst_tensoryy | 0 | N/m² | Surface tension force, yy component | Domain 1 |
| spf.Fst_tensorzy | 0 | N/m² | Surface tension force, zy component | Domain 1 |
| spf.Fst_tensorxz | 0 | N/m² | Surface tension force, xz component | Domain 1 |
| spf.Fst_tensoryz | 0 | N/m² | Surface tension force, yz component | Domain 1 |
| spf.Fst_tensorzz | 0 | N/m² | Surface tension force, zz component | Domain 1 |
| spf.rhoref | subst(material.rho,spf.fp1.minput_temperature,spf.Tref,spf.pA,spf.pref) | kg/m³ | Reference density | Domain 1 |
| spf.sr | sqrt(0.5*(4*ux^2+2*(uy+vx)^2+4*vy^2)+eps) | 1/s | Shear rate | Domain 1 |
| spf.divu | ux+vy | 1/s | Divergence of velocity field | Domain 1 |
| spf.Fx | 0 | N/m³ | Volume force, x component | Domain 1 |
| spf.Fy | 0 | N/m³ | Volume force, y component | Domain 1 |
| spf.Fz | 0 | N/m³ | Volume force, z component | Domain 1 |
| spf.U | sqrt(u^2+v^2) | m/s | Velocity magnitude | Domain 1 |
| spf.vorticityx | 0 | 1/s | Vorticity field, x component | Domain 1 |
| spf.vorticityy | 0 | 1/s | Vorticity field, y component | Domain 1 |
| spf.vorticityz | vx-uy | 1/s | Vorticity field, z component | Domain 1 |
| spf.vort_magn | sqrt(spf.vorticityx^2+spf.vorticityy^2+spf.vorticityz^2) | 1/s | Vorticity magnitude | Domain 1 |
| spf.cellRe | 0.25*spf.rho*sqrt(emetric_spatial(u-d(x,TIME),v-d(y,TIME))/emetric2_spatial)/spf.mu | 1 | Cell Reynolds number | Domain 1 |
| spf.nu | spf.mu/spf.rho | m²/s | Kinematic viscosity | Domain 1 |
| spf.betaT | 0 | 1/Pa | Isothermal compressibility coefficient | Domain 1 |
| spf.helpCont | -spf.rho*spf.divu | kg/(m³·s) | Help variable | Domain 1 |
| spf.Fgtotx | 0 | N/m³ | Gravity force, x component | Domain 1 |
| spf.Fgtoty | 0 | N/m³ | Gravity force, y component | Domain 1 |
| spf.Fgtotz | 0 | N/m³ | Gravity force, z component | Domain 1 |
| spf.mu_eff | spf.mu+spf.muT | Pa·s | Dynamic viscosity | Domain 1 |
| spf.T_stressx | spf.K_stressx-p*spf.nxmesh | N/m² | Total stress, x component | Boundaries 1–12 |
| spf.T_stressy | spf.K_stressy-p*spf.nymesh | N/m² | Total stress, y component | Boundaries 1–12 |
| spf.T_stressz | spf.K_stressz-p*spf.nzmesh | N/m² | Total stress, z component | Boundaries 1–12 |
| spf.K_stressx | spf.mu_eff*(2*ux*spf.nxmesh+(uy+vx)*spf.nymesh) | N/m² | Viscous stress, x component | Boundaries 1–12 |
| spf.K_stressy | spf.mu_eff*((vx+uy)*spf.nxmesh+2*vy*spf.nymesh) | N/m² | Viscous stress, y component | Boundaries 1–12 |
| spf.K_stressz | 0 | N/m² | Viscous stress, z component | Boundaries 1–12 |
| spf.K_stress_tensorxx | 2*spf.mu_eff*ux | N/m² | Viscous stress tensor, xx component | Domain 1 |
| spf.K_stress_tensoryx | spf.mu_eff*(vx+uy) | N/m² | Viscous stress tensor, yx component | Domain 1 |
| spf.K_stress_tensorzx | 0 | N/m² | Viscous stress tensor, zx component | Domain 1 |
| spf.K_stress_tensorxy | spf.mu_eff*(uy+vx) | N/m² | Viscous stress tensor, xy component | Domain 1 |
| spf.K_stress_tensoryy | 2*spf.mu_eff*vy | N/m² | Viscous stress tensor, yy component | Domain 1 |
| spf.K_stress_tensorzy | 0 | N/m² | Viscous stress tensor, zy component | Domain 1 |
| spf.K_stress_tensorxz | 0 | N/m² | Viscous stress tensor, xz component | Domain 1 |
| spf.K_stress_tensoryz | 0 | N/m² | Viscous stress tensor, yz component | Domain 1 |
| spf.K_stress_tensorzz | 0 | N/m² | Viscous stress tensor, zz component | Domain 1 |
| spf.K_stress_tensor_testxx | 2*spf.mu_eff*test(ux) | N/m² | Viscous stress tensor test, xx component | Domain 1 |
| spf.K_stress_tensor_testyx | spf.mu_eff*(test(vx)+test(uy)) | N/m² | Viscous stress tensor test, yx component | Domain 1 |
| spf.K_stress_tensor_testzx | 0 | N/m² | Viscous stress tensor test, zx component | Domain 1 |
| spf.K_stress_tensor_testxy | spf.mu_eff*(test(uy)+test(vx)) | N/m² | Viscous stress tensor test, xy component | Domain 1 |
| spf.K_stress_tensor_testyy | 2*spf.mu_eff*test(vy) | N/m² | Viscous stress tensor test, yy component | Domain 1 |
| spf.K_stress_tensor_testzy | 0 | N/m² | Viscous stress tensor test, zy component | Domain 1 |
| spf.K_stress_tensor_testxz | 0 | N/m² | Viscous stress tensor test, xz component | Domain 1 |
| spf.K_stress_tensor_testyz | 0 | N/m² | Viscous stress tensor test, yz component | Domain 1 |
| spf.K_stress_tensor_testzz | 0 | N/m² | Viscous stress tensor test, zz component | Domain 1 |
| spf.upwind_helpx | u-d(x,TIME) | m/s | Upwind term, x component | Domain 1 |
| spf.upwind_helpy | v-d(y,TIME) | m/s | Upwind term, y component | Domain 1 |
| spf.upwind_helpz | 0 | m/s | Upwind term, z component | Domain 1 |
| spf.isSPF | 1 | 1 | Help variable | Domain 1 |
| spf.isMPF | 0 | 1 | Help variable | Domain 1 |
| spf.tau_vdxx | 2*spf.mu*ux | Pa | Viscous stress tensor, xx component | Domain 1 |
| spf.tau_vdyx | spf.mu*(vx+uy) | Pa | Viscous stress tensor, yx component | Domain 1 |
| spf.tau_vdzx | 0 | Pa | Viscous stress tensor, zx component | Domain 1 |
| spf.tau_vdxy | spf.mu*(uy+vx) | Pa | Viscous stress tensor, xy component | Domain 1 |
| spf.tau_vdyy | 2*spf.mu*vy | Pa | Viscous stress tensor, yy component | Domain 1 |
| spf.tau_vdzy | 0 | Pa | Viscous stress tensor, zy component | Domain 1 |
| spf.tau_vdxz | 0 | Pa | Viscous stress tensor, xz component | Domain 1 |
| spf.tau_vdyz | 0 | Pa | Viscous stress tensor, yz component | Domain 1 |
| spf.tau_vdzz | 0 | Pa | Viscous stress tensor, zz component | Domain 1 |
| spf.Qvd | spf.tau_vdxx*ux+spf.tau_vdxy*uy+spf.tau_vdyx*vx+spf.tau_vdyy*vy+spf.rho*spf.ep_nn | W/m³ | Viscous dissipation | Domain 1 |
| spf.lref | 0.021000000000000005 | m | Reference length | Domain 1 |
| spf.res_k | -(spf.mu+spf.muT/spf.sigmak)*(kxx+kyy)-spf.linSCk*k-spf.Pk+spf.rho*u*kx+spf.rho*v*ky | W/m³ | Turbulent kinetic energy equation residual | Domain 1 |
| spf.res_ep | -(spf.mu+spf.muT/spf.sigmaeps)*(epxx+epyy)-spf.linSCeps*ep-spf.Peps+spf.rho*u*epx+spf.rho*v*epy | Pa/s² | Turbulent dissipation rate equation residual | Domain 1 |
| spf.res_u | px+spf.rho*u*ux+spf.rho*v*uy-(d(2*ux,x)+d(uy+vx,y))*(spf.mu+spf.muT)-spf.Fx | N/m³ | Equation residual | Domain 1 |
| spf.res_v | spf.rho*u*vx+py+spf.rho*v*vy-(d(vx+uy,x)+d(2*vy,y))*(spf.mu+spf.muT)-spf.Fy | N/m³ | Equation residual | Domain 1 |
| spf.res_p | spf.rho*spf.divu | kg/(m³·s) | Pressure equation residual | Domain 1 |

#### Shape functions

| Name | Shape function | Unit | Description | Shape frame | Selection |
| --- | --- | --- | --- | --- | --- |
| u | Lagrange (Linear) | m/s | Velocity field, x component | Spatial | Domain 1 |
| v | Lagrange (Linear) | m/s | Velocity field, y component | Spatial | Domain 1 |
| p | Lagrange (Linear) | Pa | Pressure | Spatial | Domain 1 |
| k | Lagrange (Linear) | m²/s² | Turbulent kinetic energy | Spatial | Domain 1 |
| ep | Lagrange (Linear) | m²/s³ | Turbulent dissipation rate | Spatial | Domain 1 |

#### Weak expressions

| Weak expression | Integration order | Integration frame | Selection |
| --- | --- | --- | --- |
| -test(kx)*(spf.mu+spf.muT/spf.sigmak)*kx-test(ky)*(spf.mu+spf.muT/spf.sigmak)*ky-spf.rho*(u*kx+v*ky)*test(k)+spf.Pk*test(k)+spf.linSCk*k*test(k) | 2 | Spatial | Domain 1 |
| -test(epx)*(spf.mu+spf.muT/spf.sigmaeps)*epx-test(epy)*(spf.mu+spf.muT/spf.sigmaeps)*epy-spf.rho*(u*epx+v*epy)*test(ep)+spf.Peps*test(ep)+spf.linSCeps*ep*test(ep) | 2 | Spatial | Domain 1 |
| -spf.crosswindep-spf.crosswindk | 2 | Spatial | Domain 1 |
| spf.streamlinek | 2 | Spatial | Domain 1 |
| spf.streamlineep | 2 | Spatial | Domain 1 |
| (p-spf.K_stress_tensorxx)*test(ux)-spf.K_stress_tensorxy*test(uy)-spf.K_stress_tensoryx*test(vx)+(p-spf.K_stress_tensoryy)*test(vy) | 2 | Spatial | Domain 1 |
| spf.Fx*test(u)+spf.Fy*test(v) | 2 | Spatial | Domain 1 |
| spf.rho*(-(d(u,x)*u+d(u,y)*v)*test(u)-(d(v,x)*u+d(v,y)*v)*test(v)) | 2 | Spatial | Domain 1 |
| -spf.rho*spf.divu*test(p)*spf.isSPF | 2 | Spatial | Domain 1 |
| -spf.divu*test(p)*spf.isMPF | 2 | Spatial | Domain 1 |
| spf.streamlinens | 2 | Spatial | Domain 1 |
| spf.crosswindns | 2 | Spatial | Domain 1 |

Initial Values 1

Initial Values 1

Selection

| Geometric entity level | Domain |
| --- | --- |
| Selection | Domain 1 |

Settings

| Description | Value |
| --- | --- |
| Velocity field, x component | 0 |
| Velocity field, y component | 0 |
| Velocity field, z component | 0 |
| Pressure | 0 |
| Turbulent kinetic energy | spf.kinit |
| Turbulent dissipation rate | spf.epinit |

#### Variables

| Name | Expression | Unit | Description | Selection |
| --- | --- | --- | --- | --- |
| spf.u_initx | 0 | m/s | Velocity field, x component | Domain 1 |
| spf.u_inity | 0 | m/s | Velocity field, y component | Domain 1 |
| spf.u_initz | 0 | m/s | Velocity field, z component | Domain 1 |
| spf.p_init | 0 | Pa | Pressure | Domain 1 |
| spf.k_init | spf.kinit | m²/s² | Turbulent kinetic energy | Domain 1 |
| spf.ep_init | spf.epinit | m²/s³ | Turbulent dissipation rate | Domain 1 |

Wall 1

Wall 1

Selection

| Geometric entity level | Boundary |
| --- | --- |
| Selection | Boundaries 2–11 |

Equations

Settings

| Description | Value |
| --- | --- |
| Wall condition | No slip |
| Translational velocity | Automatic from frame |
| Sliding wall | Off |
| Apply reaction terms on | Individual dependent variables |
| Use weak constraints | Off |
| Constraint method | Elemental |
| Apply wall roughness | Off |

#### Variables

| Name | Expression | Unit | Description | Selection |
| --- | --- | --- | --- | --- |
| spf.hasWF | 1 |  | Help variable | Boundaries 2–11 |
| spf.k_nn | max(k,0) | m²/s² | Regularized turbulent kinetic energy | Boundaries 2–11 |
| spf.ubndx | 0 | m/s | Velocity at boundary, x component | Boundaries 2–11 |
| spf.ubndy | 0 | m/s | Velocity at boundary, y component | Boundaries 2–11 |
| spf.ubndz | 0 | m/s | Velocity at boundary, z component | Boundaries 2–11 |
| spf.rhoFace | down(spf.rho) | kg/m³ | Density face value | Boundaries 2–11 |
| spf.c_here | 24*nojac(down(spf.mu+spf.muT)/down(1))*spf.meshVol/spf.meshVolInt | 1 | Intermediate variable | Boundaries 2–11 |
| spf.meshVol | meshvol | m |  | Boundaries 2–11 |
| spf.meshVolInt | down(meshvol) | m² | Volume of interior mesh element | Boundaries 2–11 |
| spf.umxTnFace | (spf.upwind_helpx*spf.nxmesh+spf.upwind_helpy*spf.nymesh+spf.upwind_helpz*spf.nzmesh<0)*(spf.upwind_helpx*spf.nxmesh+spf.upwind_helpy*spf.nymesh+spf.upwind_helpz*spf.nzmesh) | m/s | Relative velocity on face | Boundaries 2–11 |
| spf.upwind_ns | spf.rhoFace*spf.umxTnFace*spf.unJump*test(spf.un_here)/down(1)^2 | Pa | Upwind term | Boundaries 2–11 |
| spf.uLeakagex | 0 | m/s | Leakage velocity, x component | Boundaries 2–11 |
| spf.uLeakagey | 0 | m/s | Leakage velocity, y component | Boundaries 2–11 |
| spf.uLeakagez | 0 | m/s | Leakage velocity, z component | Boundaries 2–11 |
| spf.un_here | u*nojac(spf.nxmesh)+v*nojac(spf.nymesh) | m/s | Intermediate variable | Boundaries 2–11 |
| spf.u_herex | spf.un_here*nojac(spf.nxmesh) | m/s | Intermediate variable, x component | Boundaries 2–11 |
| spf.u_herey | spf.un_here*nojac(spf.nymesh) | m/s | Intermediate variable, y component | Boundaries 2–11 |
| spf.u_herez | spf.un_here*nojac(spf.nzmesh) | m/s | Intermediate variable, z component | Boundaries 2–11 |
| spf.un_there | (spf.ubndx+spf.uLeakagex)*nojac(spf.nxmesh)+(spf.ubndy+spf.uLeakagey)*nojac(spf.nymesh)+(spf.ubndz+spf.uLeakagez)*nojac(spf.nzmesh) | m/s | Intermediate variable | Boundaries 2–11 |
| spf.u_therex | spf.un_there*nojac(spf.nxmesh) | m/s | Intermediate variable, x component | Boundaries 2–11 |
| spf.u_therey | spf.un_there*nojac(spf.nymesh) | m/s | Intermediate variable, y component | Boundaries 2–11 |
| spf.u_therez | spf.un_there*nojac(spf.nzmesh) | m/s | Intermediate variable, z component | Boundaries 2–11 |
| spf.unJump | spf.un_here-spf.un_there | m/s | Jump in normal velocity | Boundaries 2–11 |
| spf.KStressn_avx | spf.K_stress_tensorxx*spf.nxmesh+spf.K_stress_tensorxy*spf.nymesh+spf.K_stress_tensorxz*spf.nzmesh | N/m² | Average viscous stress, x component | Boundaries 2–11 |
| spf.KStressn_avy | spf.K_stress_tensoryx*spf.nxmesh+spf.K_stress_tensoryy*spf.nymesh+spf.K_stress_tensoryz*spf.nzmesh | N/m² | Average viscous stress, y component | Boundaries 2–11 |
| spf.KStressn_avz | spf.K_stress_tensorzx*spf.nxmesh+spf.K_stress_tensorzy*spf.nymesh+spf.K_stress_tensorzz*spf.nzmesh | N/m² | Average viscous stress, z component | Boundaries 2–11 |
| spf.KStressTestn_avx | spf.K_stress_tensor_testxx*spf.nxmesh+spf.K_stress_tensor_testxy*spf.nymesh+spf.K_stress_tensor_testxz*spf.nzmesh | N/m² | Average viscous stress, x component | Boundaries 2–11 |
| spf.KStressTestn_avy | spf.K_stress_tensor_testyx*spf.nxmesh+spf.K_stress_tensor_testyy*spf.nymesh+spf.K_stress_tensor_testyz*spf.nzmesh | N/m² | Average viscous stress, y component | Boundaries 2–11 |
| spf.KStressTestn_avz | spf.K_stress_tensor_testzx*spf.nxmesh+spf.K_stress_tensor_testzy*spf.nymesh+spf.K_stress_tensor_testzz*spf.nzmesh | N/m² | Average viscous stress, z component | Boundaries 2–11 |
| spf.ujumpx | spf.u_herex-spf.u_therex | m/s | Velocity jump, x component | Boundaries 2–11 |
| spf.ujumpy | spf.u_herey-spf.u_therey | m/s | Velocity jump, y component | Boundaries 2–11 |
| spf.ujumpz | spf.u_herez-spf.u_therez | m/s | Velocity jump, z component | Boundaries 2–11 |
| spf.sigma_dg_ns | 4*spf.c_here | 1 |  | Boundaries 2–11 |
| spf.pFace | p | Pa | Pressure face value | Boundaries 2–11 |
| spf.consFlux | -spf.pFace*test(spf.un_here) | W/m² | Conservative flux | Boundaries 2–11 |
| spf.upwindCont | spf.rhoFace*spf.unJump*test(p) | kg²/(m³·s³) | Upwind term for continuity equation | Boundaries 2–11 |
| spf.Delta_wPlus | spf.d_w_plus | 1 | Wall resolution in viscous units | Boundaries 2–11 |
| spf.WRHeightExpr | -11.06+spf.d_w_plus | 1 | Wall resolution height expression | Boundaries 2–11 |
| spf.d_w_plus | nojac(max(11.06,0.5*spf.mixLParam^0.25*sqrt(spf.k_nn)*down(spf.rho)/(down(spf.mu)*down(tremetric)^0.5))) | 1 | Wall lift-off in viscous units | Boundaries 2–11 |
| spf.uPlus | log(spf.d_w_plus)/spf.kappav+spf.B | 1 | Tangential velocity in viscous units | Boundaries 2–11 |
| spf.mixLParam | spf.C_mu | 1 | Intermediate variable | Boundaries 2–11 |
| spf.ep_w | spf.mixLParam*spf.k_nn^2*nojac(down(spf.rho))/(spf.kappav*spf.d_w_plus*nojac(down(spf.mu))) | m²/s³ | Turbulent dissipation rate, (wall adjacent cells) | Boundaries 2–11 |
| spf.u_tau | nojac(max(spf.mixLParam^0.25*sqrt(spf.k_nn),sqrt(spf.u_tangx^2+spf.u_tangy^2+spf.u_tangz^2+eps)/spf.uPlus)) | m/s | Friction velocity | Boundaries 2–11 |
| spf.u_tangx | u-spf.ubndx-spf.nxmesh*((u-spf.ubndx)*spf.nxmesh+(v-spf.ubndy)*spf.nymesh-spf.ubndz*spf.nzmesh) | m/s | Velocity field, x component | Boundaries 2–11 |
| spf.u_tangy | v-spf.ubndy-spf.nymesh*((u-spf.ubndx)*spf.nxmesh+(v-spf.ubndy)*spf.nymesh-spf.ubndz*spf.nzmesh) | m/s | Velocity field, y component | Boundaries 2–11 |
| spf.u_tangz | -spf.ubndz-spf.nzmesh*((u-spf.ubndx)*spf.nxmesh+(v-spf.ubndy)*spf.nymesh-spf.ubndz*spf.nzmesh) | m/s | Velocity field, z component | Boundaries 2–11 |
| spf.delta_w | spf.d_w_plus*down(spf.mu)/(down(spf.rho)*max(spf.u_tau,sqrt(eps))) | m | Wall lift-off | Boundaries 2–11 |

#### Weak expressions

| Weak expression | Integration order | Integration frame | Selection |
| --- | --- | --- | --- |
| spf.KStressn_avx*test(spf.u_herex)+spf.KStressn_avy*test(spf.u_herey)+spf.KStressn_avz*test(spf.u_herez)+spf.KStressTestn_avx*spf.ujumpx+spf.KStressTestn_avy*spf.ujumpy+spf.KStressTestn_avz*spf.ujumpz-spf.sigma_dg_ns*spf.unJump*test(spf.un_here)+spf.upwind_ns+spf.upwindCont+spf.consFlux | 2 | Spatial | Boundaries 2–11 |
| down(spf.rho)*spf.u_tau*(-spf.u_tangx*test(u)-spf.u_tangy*test(v))/spf.uPlus | 2 | Spatial | Boundaries 2–11 |

#### Shape functions

| Constraint | Constraint force | Shape function | Selection |
| --- | --- | --- | --- |
| -ep+spf.ep_w | test(-ep) | Lagrange (Linear) | Boundaries 2–11 |

Inlet 1

Inlet 1

Selection

| Geometric entity level | Boundary |
| --- | --- |
| Selection | Boundary 1 |

Equations

Settings

| Description | Value |
| --- | --- |
| Boundary condition | Velocity |
| Velocity field componentwise | Normal inflow velocity |
| Normal inflow velocity | U |
|  | Specify turbulent length scale and intensity |
| Turbulent intensity | spf.IT_init |
| Turbulence length scale | spf.LT_init |
| Use weak constraints | Off |
| Apply reaction terms on | All physics (symmetric) |
| Constraint method | Elemental |

#### Variables

| Name | Expression | Unit | Description | Selection |
| --- | --- | --- | --- | --- |
| spf.ubndx | -nojac(spf.nxmesh)*spf.U0in | m/s | Velocity at boundary, x component | Boundary 1 |
| spf.ubndy | -nojac(spf.nymesh)*spf.U0in | m/s | Velocity at boundary, y component | Boundary 1 |
| spf.ubndz | -nojac(spf.nzmesh)*spf.U0in | m/s | Velocity at boundary, z component | Boundary 1 |
| spf.U0in | U | m/s | Normal inflow velocity | Boundary 1 |
| spf.IT | spf.IT_init | 1 | Turbulent intensity | Boundary 1 |
| spf.LT | spf.LT_init | m | Turbulence length scale | Boundary 1 |
| spf.k0 | 1.5*(spf.IT*spf.U0in)^2 | m²/s² | Turbulent kinetic energy | Boundary 1 |
| spf.ep0 | spf.C_mu^0.75*spf.k0^1.5/spf.LT | m²/s³ | Turbulent dissipation rate | Boundary 1 |
| spf.IT_init | 0.05 | 1 | Turbulent intensity | Boundary 1 |
| spf.LT_init | 0.01 | m | Turbulence length scale | Boundary 1 |
| spf.zeta0_init | 2/3 | 1 | Turbulent relative fluctuations | Boundary 1 |
| spf.alpha0_init | 1 | 1 | Elliptic blending function | Boundary 1 |

#### Shape functions

| Constraint | Constraint force | Shape function | Selection |
| --- | --- | --- | --- |
| -u+spf.ubndx | test(-u+spf.ubndx) | Lagrange (Linear) | Boundary 1 |
| -v+spf.ubndy | test(-v+spf.ubndy) | Lagrange (Linear) | Boundary 1 |
| spf.ubndz | test(spf.ubndz) |  | Boundary 1 |
| -ep+spf.ep0 | test(-ep+spf.ep0) | Lagrange (Linear) | Boundary 1 |
| -k+spf.k0 | test(-k+spf.k0) | Lagrange (Linear) | Boundary 1 |

Outlet 1

Outlet 1

Selection

| Geometric entity level | Boundary |
| --- | --- |
| Selection | Boundary 12 |

Equations

Settings

| Description | Value |
| --- | --- |
| Boundary condition | Pressure |
| Pressure | 0 |
| Normal flow | Off |
| Suppress backflow | On |
| Use weak constraints | Off |
| Apply reaction terms on | All physics (symmetric) |
| Constraint method | Elemental |

#### Variables

| Name | Expression | Unit | Description | Selection |
| --- | --- | --- | --- | --- |
| spf.rhoFace | down(spf.rho) | kg/m³ | Density face value | Boundary 12 |
| spf.c_here | 96/spf.epsilon_p | 1 | Intermediate variable | Boundary 12 |
| spf.meshVol | meshvol_spatial | m |  | Boundary 12 |
| spf.meshVolInt | down(meshvol_spatial) | m² | Volume of interior mesh element | Boundary 12 |
| spf.umxTnFace | spf.upwind_helpx*spf.nxmesh+spf.upwind_helpy*spf.nymesh+spf.upwind_helpz*spf.nzmesh | m/s | Relative velocity on face | Boundary 12 |
| spf.upwind_ns | spf.backflowPenaltyConv*spf.uNormal | Pa | Upwind term | Boundary 12 |
| spf.p0 | 0 | Pa | Pressure | Boundary 12 |
| spf.out1.Uav | 0 | m/s | Average velocity | Global |
| spf.isPressure | 1 |  | Help variable | Boundary 12 |
| spf.f0 | spf.p0+spf.uNormal*(spf.backflowPenaltyDiff-spf.backflowPenaltyConv)*(spf.uNormal<0) | N/m² | Normal stress | Boundary 12 |
| spf.uNormal | u*nojac(spf.nxmesh)+v*nojac(spf.nymesh) | m/s | Normal velocity | Boundary 12 |
| spf.backflowPenaltyDiff | spf.c_here*min((down(spf.mu)+spf.muT)*spf.meshVol/spf.meshVolInt,down(spf.rho)*abs(spf.uNormal)/down(spf.epsilon_p)) | Pa·s/m | Backflow penalty parameter, diffusive contribution | Boundary 12 |
| spf.backflowPenaltyConv | spf.rhoFace*spf.umxTnFace/spf.epsilon_p^2 | kg/(m²·s) | Backflow penalty parameter, convective contribution | Boundary 12 |
| spf.out1.Mflow | spf.out1.intFlow(spf.rho*(spf.nx*u+spf.ny*v)) | kg/s | Mass flow | Global |

#### Weak expressions

| Weak expression | Integration order | Integration frame | Selection |
| --- | --- | --- | --- |
| -spf.f0*(test(u)*spf.nxmesh+test(v)*spf.nymesh) | 2 | Spatial | Boundary 12 |

Mesh 1

Mesh statistics

| Description | Value |
| --- | --- |
| Minimum element quality | 0.1879 |
| Average element quality | 0.8123 |
| Triangular elements | 16994 |
| Quadrilateral elements | 3530 |
| Edge elements | 774 |
| Vertex elements | 12 |

Mesh 1

Size (size)

Settings

| Description | Value |
| --- | --- |
| Calibrate for | Fluid dynamics |
| Maximum element size | 0.00189 |
| Minimum element size | 8.4E-5 |
| Curvature factor | 0.3 |
| Maximum element growth rate | 1.15 |

Size 1 (size1)

Selection

| Geometric entity level | Boundary |
| --- | --- |
| Selection | Boundaries 2–11 |

Size 1

Settings

| Description | Value |
| --- | --- |
| Calibrate for | Fluid dynamics |
| Maximum element size | 0.00118 |
| Minimum element size | 1.68E-5 |
| Curvature factor | 0.25 |
| Predefined size | Finer |

Corner Refinement 1 (cr1)

Selection

| Geometric entity level | Domain |
| --- | --- |
| Selection | Domain 1 |

Corner Refinement 1

Free Triangular 1 (ftri1)

Selection

| Geometric entity level | Remaining |
| --- | --- |

Free Triangular 1

Boundary Layers 1 (bl1)

Selection

| Geometric entity level | Domain |
| --- | --- |
| Selection | Domain 1 |

Boundary Layers 1

Settings

| Description | Value |
| --- | --- |
| Handling of sharp corners | Trimming |

#### Boundary Layer Properties 1 (blp1)

Selection

| Geometric entity level | Boundary |
| --- | --- |
| Selection | Boundaries 2–11 |

Boundary Layer Properties 1

Settings

| Description | Value |
| --- | --- |
| Number of boundary layers | 5 |
| Thickness adjustment factor | 2.5 |

Study 1

Computation information

| Computation time | 2 min 4 s |
| --- | --- |
| CPU | Intel(R) Core(TM) i7-2640M CPU @ 2.80GHz, 2 cores |
| Operating system | Windows 7 |

Stationary

Study settings

| Description | Value |
| --- | --- |
| Include geometric nonlinearity | Off |

Physics and variables selection

| Physics interface | Discretization |
| --- | --- |
| Turbulent Flow, k-ε (spf) | physics |

Mesh selection

| Geometry | Mesh |
| --- | --- |
| Geometry 1 (geom1) | mesh1 |

Solver Configurations

Solution 1

#### Compile Equations: Stationary (st1)

Study and step

| Description | Value |
| --- | --- |
| Use study | Study 1 |
| Use study step | Stationary |

Log

<---- Compile Equations: Stationary in Study 1/Solution 1 (sol1) ---------------

Started at 9-Jan-2025 03:32:35.

Geometry shape order: Linear

Running on Intel(R) Core(TM) i7-2640M CPU at 2.80 GHz.

Using 2 cores on 1 socket.

Available memory: 4.00 GB.

Time: 2 s.

Physical memory: 876 MB

Virtual memory: 1014 MB

Ended at 9-Jan-2025 03:32:36.

----- Compile Equations: Stationary in Study 1/Solution 1 (sol1) -------------->

#### Dependent Variables 1 (v1)

General

| Description | Value |
| --- | --- |
| Defined by study step | Stationary |

##### Turbulent dissipation rate (comp1.ep) (comp1_ep)

General

| Description | Value |
| --- | --- |
| Field components | comp1.ep |

##### Turbulent kinetic energy (comp1.k) (comp1_k)

General

| Description | Value |
| --- | --- |
| Field components | comp1.k |

##### Pressure (comp1.p) (comp1_p)

General

| Description | Value |
| --- | --- |
| Field components | comp1.p |

##### Velocity field (comp1.u) (comp1_u)

General

| Description | Value |
| --- | --- |
| Field components | {comp1.u, comp1.v} |

#### Stationary Solver 1 (s1)

General

| Description | Value |
| --- | --- |
| Defined by study step | Stationary |

Log

0.13

Segregated solver iteration 9.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.75   0.5000000        0.75   18    9    9   6e-011 8.8e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.063    3.8e+004   0.3500000       0.063   34   25   25 3.5e-013 6.4e-016

2       0.061    3.8e+004   0.3500000       0.061   35   26   26 3.4e-013 5.8e-016

3        0.06    3.8e+004   0.3500000        0.06   36   27   27 2.7e-013   6e-016

Solution error estimates for segregated groups

0.38, 0.065

Residual error estimates for segregated groups

2.5e+003, 3.7e+004

Pseudo time-stepping CFL-ratio:

0.11

Segregated solver iteration 10.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.71   0.5000000        0.71   20   10   10 3.3e-011 8.6e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.078    3.8e+004   0.3500000       0.078   38   28   28   3e-013 7.4e-016

2       0.075    3.8e+004   0.3500000       0.075   39   29   29 3.4e-013 6.9e-016

3       0.073    3.8e+004   0.3500000       0.073   40   30   30 3.7e-013 6.5e-016

Solution error estimates for segregated groups

0.35, 0.08

Residual error estimates for segregated groups

1.8e+003, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.15

Segregated solver iteration 11.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.51   0.5000000        0.51   22   11   11   4e-011   7e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.068    3.8e+004   0.3500000       0.068   42   31   31 2.9e-013 6.4e-016

2       0.066    3.8e+004   0.3500000       0.066   43   32   32 2.9e-013 6.1e-016

3       0.064    3.8e+004   0.3500000       0.064   44   33   33 3.1e-013 5.8e-016

Solution error estimates for segregated groups

0.25, 0.07

Residual error estimates for segregated groups

1.5e+003, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.14

Segregated solver iteration 12.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.35   0.5000000        0.35   24   12   12 4.9e-011 4.9e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.075    3.8e+004   0.3500000       0.075   46   34   34 3.3e-013 6.8e-016

2       0.073    3.8e+004   0.3500000       0.073   47   35   35 3.3e-013 6.7e-016

3       0.071    3.8e+004   0.3500000       0.071   48   36   36   3e-013 6.6e-016

Solution error estimates for segregated groups

0.18, 0.077

Residual error estimates for segregated groups

1.3e+003, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.16

Segregated solver iteration 13.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.23   0.5000000        0.23   26   13   13   5e-011 3.1e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.08    3.9e+004   0.3500000        0.08   50   37   37 3.4e-013 7.4e-016

2       0.077    3.9e+004   0.3500000       0.077   51   38   38 3.6e-013 6.6e-016

3       0.075    3.9e+004   0.3500000       0.075   52   39   39 3.3e-013 6.3e-016

Solution error estimates for segregated groups

0.11, 0.082

Residual error estimates for segregated groups

1.2e+003, 3.9e+004

Pseudo time-stepping CFL-ratio:

0.18

Segregated solver iteration 14.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.15   0.5000000        0.15   28   14   14 2.8e-011 1.9e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.086    3.9e+004   0.3500000       0.086   54   40   40   4e-013 7.3e-016

2       0.083    3.9e+004   0.3500000       0.083   55   41   41   4e-013 7.1e-016

3        0.08    3.9e+004   0.3500000        0.08   56   42   42 3.9e-013 6.8e-016

Solution error estimates for segregated groups

0.073, 0.089

Residual error estimates for segregated groups

1.1e+003, 3.9e+004

Pseudo time-stepping CFL-ratio:

0.19

Segregated solver iteration 15.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.11   0.5000000        0.11   30   15   15 1.6e-011 1.3e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.089    3.9e+004   0.3500000       0.089   58   43   43 3.9e-013 7.2e-016

2       0.085    3.9e+004   0.3500000       0.085   59   44   44 3.4e-013 6.8e-016

3       0.082    3.9e+004   0.3500000       0.082   60   45   45 4.3e-013 6.1e-016

Solution error estimates for segregated groups

0.055, 0.092

Residual error estimates for segregated groups

1.1e+003, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.21

Segregated solver iteration 16.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.096   0.5000000       0.096   32   16   16 1.1e-011 1.1e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.087    3.9e+004   0.3500000       0.087   62   46   46 3.6e-013 6.1e-016

2       0.084    3.9e+004   0.3500000       0.084   63   47   47 4.6e-013 6.1e-016

3       0.081    3.8e+004   0.3500000       0.081   64   48   48 4.2e-013 5.4e-016

Solution error estimates for segregated groups

0.048, 0.09

Residual error estimates for segregated groups

1e+003, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.22

Segregated solver iteration 17.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.09   0.5000000        0.09   34   17   17 3.3e-012 1.1e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.084    3.9e+004   0.3500000       0.084   66   49   49 3.6e-013 5.8e-016

2       0.081    3.9e+004   0.3500000       0.081   67   50   50 3.8e-013 5.5e-016

3       0.079    3.8e+004   0.3500000       0.079   68   51   51   4e-013 5.1e-016

Solution error estimates for segregated groups

0.045, 0.088

Residual error estimates for segregated groups

9.3e+002, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.22

Segregated solver iteration 18.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.085   0.5000000       0.085   36   18   18 9.2e-013 1.1e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.081    3.9e+004   0.3500000       0.081   70   52   52 3.3e-013 5.3e-016

2       0.079    3.9e+004   0.3500000       0.079   71   53   53 3.9e-013 4.9e-016

3       0.076    3.8e+004   0.3500000       0.076   72   54   54 3.5e-013 4.8e-016

Solution error estimates for segregated groups

0.043, 0.085

Residual error estimates for segregated groups

8.6e+002, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.23

Segregated solver iteration 19.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.081   0.5000000       0.081   38   19   19 1.1e-012 1.2e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.079    3.9e+004   0.3500000       0.079   74   55   55 3.3e-013 4.9e-016

2       0.076    3.9e+004   0.3500000       0.076   75   56   56 3.4e-013 4.7e-016

3       0.074    3.8e+004   0.3500000       0.074   76   57   57 3.1e-013 4.3e-016

Solution error estimates for segregated groups

0.04, 0.083

Residual error estimates for segregated groups

8e+002, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.23

Segregated solver iteration 20.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.076   0.5000000       0.076   40   20   20 2.3e-012 1.2e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.077    3.9e+004   0.3500000       0.077   78   58   58 3.3e-013 4.6e-016

2       0.074    3.8e+004   0.3500000       0.074   79   59   59 3.5e-013 4.2e-016

3       0.072    3.8e+004   0.3500000       0.072   80   60   60 3.9e-013 4.5e-016

Solution error estimates for segregated groups

0.038, 0.08

Residual error estimates for segregated groups

7.4e+002, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.24

Segregated solver iteration 21.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.072   0.5000000       0.072   42   21   21 3.2e-012 1.3e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.075    3.9e+004   0.3500000       0.075   82   61   61 3.1e-013 4.7e-016

2       0.072    3.8e+004   0.3500000       0.072   83   62   62 3.2e-013 4.4e-016

3        0.07    3.8e+004   0.3500000        0.07   84   63   63 4.4e-013 4.7e-016

Solution error estimates for segregated groups

0.036, 0.078

Residual error estimates for segregated groups

6.8e+002, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.24

Segregated solver iteration 22.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.068   0.5000000       0.068   44   22   22 3.1e-012 1.3e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.073    3.8e+004   0.3500000       0.073   86   64   64 3.4e-013 4.7e-016

2        0.07    3.8e+004   0.3500000        0.07   87   65   65 3.9e-013 5.2e-016

3       0.067    3.8e+004   0.3500000       0.067   88   66   66   3e-013 4.1e-016

Solution error estimates for segregated groups

0.034, 0.075

Residual error estimates for segregated groups

6.3e+002, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.25

Segregated solver iteration 23.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.064   0.5000000       0.064   46   23   23 8.4e-012 1.4e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.07    3.8e+004   0.3500000        0.07   90   67   67 3.9e-013 5.7e-016

2       0.067    3.8e+004   0.3500000       0.067   91   68   68 3.8e-013 4.4e-016

3       0.064    3.8e+004   0.3500000       0.064   92   69   69 3.8e-013 4.5e-016

Solution error estimates for segregated groups

0.032, 0.072

Residual error estimates for segregated groups

5.9e+002, 3.8e+004

Pseudo time-stepping CFL-ratio:

0.26

Segregated solver iteration 24.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.061   0.5000000       0.061   48   24   24 7.2e-012 1.5e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.067    3.8e+004   0.3500000       0.067   94   70   70 3.8e-013 4.8e-016

2       0.064    3.8e+004   0.3500000       0.064   95   71   71 3.7e-013 4.5e-016

3       0.061    3.8e+004   0.3500000       0.061   96   72   72 3.3e-013 4.5e-016

Solution error estimates for segregated groups

0.03, 0.069

Residual error estimates for segregated groups

5.6e+002, 3.7e+004

Pseudo time-stepping CFL-ratio:

0.26

Segregated solver iteration 25.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.058   0.5000000       0.058   50   25   25 4.3e-012 1.5e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.064    3.8e+004   0.3500000       0.064   98   73   73 3.8e-013 4.8e-016

2        0.06    3.7e+004   0.3500000        0.06   99   74   74 3.8e-013 4.4e-016

3       0.056    3.7e+004   0.3500000       0.056  100   75   75 4.4e-013 4.5e-016

Solution error estimates for segregated groups

0.029, 0.064

Residual error estimates for segregated groups

5.3e+002, 3.7e+004

Pseudo time-stepping CFL-ratio:

0.27

Segregated solver iteration 26.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.054   0.5000000       0.054   52   26   26 1.1e-011 1.6e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.059    3.7e+004   0.3500000       0.059  102   76   76 4.2e-013 5.2e-016

2       0.055    3.7e+004   0.3500000       0.055  103   77   77 4.1e-013 4.6e-016

3       0.052    3.7e+004   0.3500000       0.052  104   78   78   4e-013 4.7e-016

Solution error estimates for segregated groups

0.027, 0.059

Residual error estimates for segregated groups

5.1e+002, 3.7e+004

Pseudo time-stepping CFL-ratio:

0.28

Segregated solver iteration 27.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.052   0.5000000       0.052   54   27   27 7.7e-012 1.6e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.055    3.7e+004   0.3500000       0.055  106   79   79 3.9e-013 4.7e-016

2       0.051    3.7e+004   0.3500000       0.051  107   80   80 3.2e-013 4.3e-016

3       0.047    3.6e+004   0.3500000       0.047  108   81   81   4e-013 4.2e-016

Solution error estimates for segregated groups

0.026, 0.054

Residual error estimates for segregated groups

4.9e+002, 3.6e+004

Pseudo time-stepping CFL-ratio:

0.29

Segregated solver iteration 28.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.049   0.5000000       0.049   56   28   28 5.7e-012 1.7e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.05    3.6e+004   0.3500000        0.05  110   82   82 3.4e-013 4.3e-016

2       0.046    3.6e+004   0.3500000       0.046  111   83   83 4.3e-013 4.5e-016

3       0.043    3.6e+004   0.3500000       0.043  112   84   84 3.7e-013 4.3e-016

Solution error estimates for segregated groups

0.025, 0.049

Residual error estimates for segregated groups

4.7e+002, 3.5e+004

Pseudo time-stepping CFL-ratio:

0.3

Segregated solver iteration 29.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.048   0.5000000       0.048   58   29   29   1e-012 1.7e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.045    3.6e+004   0.3500000       0.045  114   85   85 4.6e-013 4.5e-016

2       0.041    3.5e+004   0.3500000       0.041  115   86   86 4.4e-013 4.4e-016

3       0.038    3.5e+004   0.3500000       0.038  116   87   87 3.6e-013 4.3e-016

Solution error estimates for segregated groups

0.024, 0.044

Residual error estimates for segregated groups

4.5e+002, 3.5e+004

Pseudo time-stepping CFL-ratio:

0.31

Segregated solver iteration 30.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.048   0.5000000       0.048   60   30   30   3e-012 1.8e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.04    3.5e+004   0.3500000        0.04  118   88   88 4.3e-013   4e-016

2       0.037    3.5e+004   0.3500000       0.037  119   89   89 4.1e-013   4e-016

3       0.034    3.4e+004   0.3500000       0.034  120   90   90 3.5e-013 3.6e-016

Solution error estimates for segregated groups

0.024, 0.04

Residual error estimates for segregated groups

4.2e+002, 3.4e+004

Pseudo time-stepping CFL-ratio:

0.32

Segregated solver iteration 31.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.048   0.5000000       0.048   62   31   31 1.1e-012 1.9e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.036    3.4e+004   0.3500000       0.036  122   91   91 3.6e-013 3.4e-016

2       0.032    3.4e+004   0.3500000       0.032  123   92   92 3.5e-013 3.5e-016

3        0.03    3.4e+004   0.3500000        0.03  124   93   93   4e-013 3.5e-016

Solution error estimates for segregated groups

0.024, 0.035

Residual error estimates for segregated groups

3.9e+002, 3.4e+004

Pseudo time-stepping CFL-ratio:

0.33

Segregated solver iteration 32.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.049   0.5000000       0.049   64   32   32 1.2e-012   2e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.031    3.4e+004   0.3500000       0.031  126   94   94   4e-013 2.9e-016

2       0.028    3.3e+004   0.3500000       0.028  127   95   95 3.5e-013 3.3e-016

3       0.025    3.3e+004   0.3500000       0.025  128   96   96 3.2e-013 3.6e-016

Solution error estimates for segregated groups

0.024, 0.03

Residual error estimates for segregated groups

3.5e+002, 3.3e+004

Pseudo time-stepping CFL-ratio:

0.35

Segregated solver iteration 33.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.048   0.5000000       0.048   66   33   33 2.2e-012 2.2e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.027    3.3e+004   0.3500000       0.027  130   97   97 4.4e-013 2.7e-016

2       0.023    3.3e+004   0.3500000       0.023  131   98   98 3.4e-013 2.9e-016

3       0.021    3.3e+004   0.3500000       0.021  132   99   99   3e-013 2.3e-016

Solution error estimates for segregated groups

0.024, 0.025

Residual error estimates for segregated groups

3.1e+002, 3.3e+004

Pseudo time-stepping CFL-ratio:

0.36

Segregated solver iteration 34.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.046   0.5000000       0.046   68   34   34 1.2e-012 2.4e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.023    3.3e+004   0.3500000       0.023  134  100  100   4e-013 2.7e-016

2       0.019    3.2e+004   0.3500000       0.019  135  101  101 3.5e-013 3.1e-016

3       0.016    3.2e+004   0.3500000       0.016  136  102  102 3.2e-013 3.8e-016

Solution error estimates for segregated groups

0.023, 0.021

Residual error estimates for segregated groups

2.7e+002, 3.2e+004

Pseudo time-stepping CFL-ratio:

0.37

Segregated solver iteration 35.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.043   0.5000000       0.043   70   35   35 8.5e-013 2.6e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.02    3.2e+004   0.3500000        0.02  138  103  103 3.8e-013 2.5e-016

2       0.016    3.2e+004   0.3500000       0.016  139  104  104 3.3e-013 2.6e-016

3       0.013    3.2e+004   0.3500000       0.013  140  105  105 1.9e-013 1.9e-016

Solution error estimates for segregated groups

0.021, 0.017

Residual error estimates for segregated groups

2.2e+002, 3.2e+004

Pseudo time-stepping CFL-ratio:

0.39

Segregated solver iteration 36.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.037   0.5000000       0.037   72   36   36 1.8e-012 2.7e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.018    3.2e+004   0.3500000       0.018  142  106  106 2.5e-013 2.2e-016

2       0.014    3.2e+004   0.3500000       0.014  143  107  107   2e-013 1.9e-016

3        0.01    3.2e+004   0.3500000        0.01  144  108  108 1.6e-013 1.4e-016

Solution error estimates for segregated groups

0.018, 0.015

Residual error estimates for segregated groups

1.7e+002, 3.2e+004

Pseudo time-stepping CFL-ratio:

0.41

Segregated solver iteration 37.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1        0.03   0.5000000        0.03   74   37   37 9.3e-013 2.9e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.017    3.2e+004   0.3500000       0.017  146  109  109 2.7e-013 2.7e-016

2       0.012    3.2e+004   0.3500000       0.012  147  110  110 1.7e-013 2.2e-016

3      0.0084    3.2e+004   0.3500000      0.0084  148  111  111 1.5e-013 1.5e-016

Solution error estimates for segregated groups

0.015, 0.013

Residual error estimates for segregated groups

1.3e+002, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.43

Segregated solver iteration 38.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.023   0.5000000       0.023   76   38   38 8.7e-013 3.1e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.015    3.2e+004   0.3500000       0.015  150  112  112   2e-013 1.8e-016

2        0.01    3.1e+004   0.3500000        0.01  151  113  113 1.9e-013 1.9e-016

3      0.0071    3.1e+004   0.3500000      0.0071  152  114  114 1.5e-013 1.7e-016

Solution error estimates for segregated groups

0.012, 0.011

Residual error estimates for segregated groups

87, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.45

Segregated solver iteration 39.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.017   0.5000000       0.017   78   39   39 1.1e-012 3.5e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.012    3.1e+004   0.3500000       0.012  154  115  115   2e-013 2.2e-016

2      0.0084    3.1e+004   0.3500000      0.0084  155  116  116 1.7e-013 2.1e-016

3      0.0057    3.1e+004   0.3500000      0.0057  156  117  117 1.5e-013 2.6e-016

Solution error estimates for segregated groups

0.0085, 0.009

Residual error estimates for segregated groups

57, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.48

Segregated solver iteration 40.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.012   0.5000000       0.012   80   40   40 1.2e-012   4e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0091    3.1e+004   0.3500000      0.0091  158  118  118 1.7e-013 1.6e-016

2      0.0063    3.1e+004   0.3500000      0.0063  159  119  119 1.4e-013 1.6e-016

3      0.0042    3.1e+004   0.3500000      0.0042  160  120  120 9.2e-014   1e-016

Solution error estimates for segregated groups

0.0061, 0.0068

Residual error estimates for segregated groups

35, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.51

Segregated solver iteration 41.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1       0.009   0.5000000       0.009   82   41   41 2.7e-012 5.3e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0065    3.1e+004   0.3500000      0.0065  162  121  121 1.9e-013 2.4e-016

2      0.0043    3.1e+004   0.3500000      0.0043  163  122  122 1.3e-013 2.8e-016

3      0.0029    3.1e+004   0.3500000      0.0029  164  123  123 1.5e-013 1.9e-016

Solution error estimates for segregated groups

0.0045, 0.0047

Residual error estimates for segregated groups

22, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.55

Segregated solver iteration 42.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0063   0.5000000      0.0063   84   42   42   8e-012 6.3e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0044    3.1e+004   0.3500000      0.0044  166  124  124 1.8e-013 2.3e-016

2      0.0028    3.1e+004   0.3500000      0.0028  167  125  125 1.9e-013 2.6e-016

3      0.0019    3.1e+004   0.3500000      0.0019  168  126  126 1.2e-013   2e-016

Solution error estimates for segregated groups

0.0032, 0.0031

Residual error estimates for segregated groups

15, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.59

Segregated solver iteration 43.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0034   0.5000000      0.0034   86   43   43 5.7e-012 5.5e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0025    3.1e+004   0.3500000      0.0025  170  127  127 1.2e-013 1.3e-016

2      0.0016    3.1e+004   0.3500000      0.0016  171  128  128 2.2e-013 3.2e-016

3      0.0012    3.1e+004   0.3500000      0.0012  172  129  129 1.2e-013   2e-016

Solution error estimates for segregated groups

0.0017, 0.0018

Residual error estimates for segregated groups

10, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.63

Segregated solver iteration 44.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0023   0.5000000      0.0023   88   44   44 2.3e-011 3.5e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0013    3.1e+004   0.3500000      0.0013  174  130  130 1.5e-013 2.9e-016

2     0.00086    3.1e+004   0.3500000     0.00086  175  131  131 1.6e-013 2.8e-016

Solution error estimates for segregated groups

0.0012, 0.00084

Residual error estimates for segregated groups

5.7, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.7

Segregated solver iteration 45.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0032   0.5000000      0.0032   90   45   45 3.4e-011 1.1e-013

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0016    3.1e+004   0.3500000      0.0016  177  132  132 1.6e-013   3e-016

2     0.00096    3.1e+004   0.3500000     0.00096  178  133  133 1.8e-013 3.8e-016

Solution error estimates for segregated groups

0.0016, 0.0011

Residual error estimates for segregated groups

3, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.76

Segregated solver iteration 46.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1     0.00032   0.5000000     0.00032   92   46   46 2.2e-012 1.8e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1     0.00064    3.1e+004   0.3500000     0.00064  180  134  134 5.8e-014 1.4e-016

Solution error estimates for segregated groups

0.00016, 0.00042

Residual error estimates for segregated groups

1.7, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.76

Segregated solver iteration 47.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0024   0.5000000      0.0024   94   47   47 6.4e-011 2.5e-013

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0012    3.1e+004   0.3500000      0.0012  182  135  135 1.1e-013 1.8e-016

2     0.00071    3.1e+004   0.3500000     0.00071  183  136  136 9.2e-014 1.6e-016

Solution error estimates for segregated groups

0.0012, 0.00077

Residual error estimates for segregated groups

1.2, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.91

Segregated solver iteration 48.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1     0.00064   0.5000000     0.00064   96   48   48 5.6e-011 8.4e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1     0.00066    3.1e+004   0.3500000     0.00066  185  137  137 1.4e-013 2.7e-016

Solution error estimates for segregated groups

0.00032, 0.00043

Residual error estimates for segregated groups

1, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.83

Segregated solver iteration 49.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1     0.00059   0.5000000     0.00059   98   49   49 2.8e-011 9.3e-014

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1     0.00014    3.1e+004   0.3500000     0.00014  187  138  138 7.8e-014 1.4e-016

Solution error estimates for segregated groups

0.00029, 9.2e-005

Residual error estimates for segregated groups

0.65, 3.1e+004

Pseudo time-stepping CFL-ratio:

0.94

Segregated solver iteration 50.

Velocity u, Pressure p

Iter      SolEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1      0.0013   0.5000000      0.0013  100   50   50 1.8e-012 2.8e-013

Turbulence variables

Iter      SolEst      ResEst     Damping    Stepsize #Res #Jac #Sol   LinErr   LinRes

1     0.00063    3.1e+004   0.3500000     0.00063  189  139  139 9.8e-014 1.6e-016

Solution error estimates for segregated groups

0.00067, 0.00041

Residual error estimates for segregated groups

0.66, 3.1e+004

Pseudo time-stepping CFL-ratio:

1

Solution time: 122 s. (2 minutes, 2 seconds)

Physical memory: 1.13 GB

Virtual memory: 1.28 GB

Ended at 9-Jan-2025 03:34:39.

----- Stationary Solver 1 in Study 1/Solution 1 (sol1) ------------------------>

##### Segregated 1 (se1)

General

| Description | Value |
| --- | --- |
| Maximum number of iterations | 300 |
| Stabilization and acceleration | Pseudo time-stepping |
| Initial CFL number | 3 |

###### Velocity u, Pressure p (ss1)

General

| Description | Value |
| --- | --- |
| Variables | {Velocity field (comp1.u), Pressure (comp1.p)} |
| Linear solver | Direct 1 |

Method and termination

| Description | Value |
| --- | --- |
| Damping factor | 0.5 |

###### Turbulence variables (ss2)

General

| Description | Value |
| --- | --- |
| Variables | {Turbulent kinetic energy (comp1.k), Turbulent dissipation rate (comp1.ep)} |
| Linear solver | Direct 2 |

Method and termination

| Description | Value |
| --- | --- |
| Damping factor | 0.35 |
| Termination technique | Iterations or tolerance |
| Number of iterations | 3 |
| Tolerance factor | 1 |

###### Lower Limit 1 (ll1)

Lower limit

| Description | Value |
| --- | --- |
| Lower limits (field variables) | comp1.k 0 comp1.ep 0 |

##### Direct 1 (d1)

General

| Description | Value |
| --- | --- |
| Solver | PARDISO |

##### Direct 2 (d2)

General

| Description | Value |
| --- | --- |
| Solver | PARDISO |

Results

Data Sets

Study 1/Solution 1

Solution

| Description | Value |
| --- | --- |
| Solution | Solution 1 |
| Component | Save Point Geometry 1 |

Data set: Study 1/Solution 1

Exterior Walls

Data

| Description | Value |
| --- | --- |
| Data set | Study 1/Solution 1 |

Plot Groups

Velocity (spf)

Surface: Velocity magnitude (m/s) UGWOKE KENECHUKWU PERFECTUS 2022/241806 Contour: Velocity magnitude (m/s) UGWOKE KENECHUKWU PERFECTUS 2022/241806 Streamline: Velocity field UGWOKE KENECHUKWU PERFECTUS 2022/241806 Arrow Surface: Velocity field UGWOKE KENECHUKWU PERFECTUS 2022/241806

Pressure (spf)

Contour: Pressure (Pa) Surface: Pressure (Pa)

Wall Resolution (spf)

Line: Wall resolution in viscous units (1)